# ECE 485 – CPX - Stage 2               5-cycle Pipelined Architecture – NOP controlled

## Overview

For the ECE 485 CPX, you will be implementing a limited version of the RISC-V 5-cycle pipeline architecture discussed in Appendix C of your Hennessey and Patterson textbook. You will evolve this design in 4 stages:

Stage 1: 5-cycle Multicycle Architecture: convert the single cycle RISC-V architecture to a 5-cycle multicycle architecture (IF-ID-EX-MEM-WB) that is NOT pipelined.  Each instruction will take 5 cycles to complete before the next instruction can begin. See Figure C.18

Stage 2: 5-cycle Pipelined Architecture: Add intermediate registers (IF_ID, ID_EX, EX_MEM, MEM_WB) to the Stage 1 architecture, so it is now pipelined.  However, this stage does NOT have hazard detection and does NOT stall to prevent hazards.  You will need to add NOPs to the code to ensure proper execution. See Figure C.19

Stage 3: 5-cycle Pipelined Architecture with Stall mitigation: convert the stage 2 architecture to a pipelined architecture that has hazard detection and stalls appropriately to ensure proper execution, but does NOT have forwarding.

Stage 4: 5-cycle Pipelined Architecture with Forwarding: convert the stage 3 architecture to a pipelined architecture that has hazard detection and stalls appropriately, plus has forwarding and branch decision moved to the ID stage, similar to Figure C.25.

The only RISC-V instructions implemented will be ADD, ADDI, SUBI, LW, SW, NOP, J, and BNE.  You will also implement a custom instruction called Load_Addr, which is similar to LA, but only loads the address 0x10000000. Details about these instructions are in a later section.

The memory map for this architecture assumes the program memory starts at address 0x00000000, and the data memory (for the Array), starts at address 0x10000000

Program to Execute

The primary program to be executed for these 4 stages is from the RISC-V exercise:

addi      x5, x0, 9       # x5 = number of values in array

load_addr x6, array       # x6 = address of array 0X10000000

lw        x7, 0(x6)       # x7 = initial value

loop:   addi      x6, x6, 4

lw        x10, 0(x6)

add       x7, x10, x7

subi      x5, x5, 1

bne       x5, x0, loop

done:   j         done            # infinite loop (breakpoint)

#   initialize data in the array [assumed to start at 0x10000000]

array:  .word 0x5, 0x4, 0x10, 0x3, 0x12, 0x1, 0x7, 0x4, 0x8, 0x2

## Block Diagram

Stage 2 design is based on Figure C.19 from your textbook.  A draft of the design is given to you in RISC-V stage 2.pptx:

## GitHub

You are also given starter-code at the GitHub repo

https://github.com/GeorgeYork/ece485_cadet_cpx_stage2

which includes:

| riscv_pipeline.vhdl: | the top level |
| --- | --- |
| pc_live.vhdl | updates new program counter [do not change] |
| pipeline_registers.vhdl | Temporary registers between each state (labeled with prefix IF_ID, ID_EX, EX_MEM, MEM_WB) to move an instruction from state to state. |
| instr_mem.vhdl | instruction memory. Holds assembly program to execute |
| reg_file.vhdl | The 32 32-bit Registers [do not change] |
| control_unit.vhdl | Given the instruction OPCODE, outputs the control signals reg_write, mem_read, mem_write, alu_src, branch, load_addr, jump [do not change] |
| immediate_generator.vhdl | Given the instruction, pulls out and sign extends (to 32-bits) the immediate value for each type of instruction. [do not change] |
| alu_control.vhdl | Given funct7 and funct3 for an instruction, outputs the alu_op (alu operation) to perform |
| alu.vhdl | Performs the arithmetic logic unit operation (ADD, SUB, etc) on two operands. |
| data_mem.vhdl | The data memory. Holds the array of values to be summed, starting at address 0x10000000 [do not change] |
| tb_riscv_pipeline.vhdl | Test bench to test overall architecture [do not need to change] |
| tb_riscv_pipeline_behav.wcfg | Waveform configuration file, set up to have the required signals in the simulation plot for the main program test |
| RISC-V stage 2.pptx | Draft block diagram of stage 2 |

Fork and Clone and open the project ece485_cadet_cpx_stage2.

## Details of Instructions

| ADD | R-Type Instruction add rd, rs1, rs2 # rd = rs1 + rs2 rd: destination register (where the result is stored) rs1: first source register rs2: second source register Instruction Format: “<7-bit funct7> <5-bit rs2> <5-bit rs1> <3-bit funct3> <5-bit rd> <7-bit opcode>” funct7= “0000000” funct3= “000” Opcode= “0110011” |
| --- | --- |
| ADDI | I-Type Instruction addi rd, rs1, imm # rd = rs1 + imm rd: destination register rs1: source register imm: immediate, sign extended to 32-bit Instruction Format: “<12-bit imm> <5-bit rs1> <3-bit funct3> <5-bit rd> <7-bit opcode>” funct3= “000” Opcode= “0010011” |
| LW | I-Type Instruction: Load Word from Memory lw rd, imm(rs1) # rd = memory[rs1 + imm] rd: destination register rs1: register containing base address (or pointer) imm: immediate, sign extended to 32-bit; offset to be added to pointer Instruction Format: “<12-bit imm> <5-bit rs1> <3-bit funct3> <5-bit rd> <7-bit opcode>” funct3= “010” Opcode= “0000011” |
| SW | Not implementing this year |
| BNE | B-Type Instruction: bne rs1, rs2, imm # PC = PC + 4 + imm, if rs1 is not equal to rs2 imm: immediate, sign extended to 32-bit; offset to be added to PC + 4 Instruction Format: “<imm[11]> <imm[9:4]> <5-bit rs2> <5-bit rs1> <3-bit funct3> <imm[3:0]> <imm[10]><7-bit opcode>” funct3= “001” Opcode= “1100011”. So the final PC = PC + 4 + (sign extend)(imm[11:0] & “00”) Note: for our implementation, imm[11:0] points to a word [not a byte or 16-bit] |

Custom Instructions [replacing Psuedo-instructions]:

| J | In the RISC-V architecture, J (jump to label) is a pseudo-instruction, where j label is replaced with jal rd, imm # jump to PC = PC + 4 + (sign extend) imm You will add a custom J instruction to your implementation. J-Type Instruction: j imm # PC = PC + 4 + imm imm: immediate, sign extended to 32-bit; offset to be added to PC + 4 Instruction Format: “<imm[20]> <imm[10:1]> <imm[11]> <imm[19:12]><5-bit rd> <7-bit opcode>” rd = not used “00000” Opcode= “1101111”. So the final PC = PC + 4 + (sign extend)(imm[20:1] & “00”) Note: for our implementation, imm[20:1] points to a word [not a byte or 16-bit] |
| --- | --- |
| SUBI | In the RISC-V architecture, SUBI (subtract immediate) is a pseudo-instruction, where subi rd, rs1, imm # rd = rs1 - imm is replaced with addi rd rs1, imm #. rd = rs1 + -imm You will add a custom SUBI instruction to your implementation. Instruction Format: “<12-bit imm> <5-bit rs1> <3-bit funct3> <5-bit rd> <7-bit opcode>” funct3= “001” Opcode= “0010011” |
| Load_Addr | In the RISC-V architecture, there is a LA pseudo-instruction, which loads an address in a register to be used as a pointer to a data array. Since we cannot have a load with a 32-bit immediate value, this pseudo-instruction is replace with two instructions to create the 32-bit value: auipc x5, offset[31:12] addi x5, x5, offset[11:0] Instead of doing this, we will create a custom instruction called load_addr, which always loads the fixed value of our array 0x10000000. Instruction Format: “<20-bit don’t care> <5-bit rd> <7-bit opcode>” rd = destination register Opcode= “0010111” |
| NOP: | In the RISC-V architecture, NOP is actually a pseudo-instruction, which is replaced by an instruction that does nothing, addi x0, x0, 0. In stage 2, you will implement a NOP instruction, but instead of using the ADDI, you will use the 32-bit instruction “0x00000000”, and when detected, your system will set all the control signals to zero (reg_write <= '0'; alu_src <= '0'; mem_read <= '0'; mem_write <= '0'; branch <= '0'; jump <= '0'; load_addr <= '0'; see control_unit.vhdl |

Tasks

By Taps Lesson 28, complete these tasks for Stage 2. For Stage 2, you will implement a 5-stage pipeline, using registers IF_ID, ID_EX, EX_MEM, MEM_WB to migrate each instruction from state to state; therefore the “state_type” [FETCH, DECODE, EXECUTE, MEMORY, WRITEBACK] will no longer be used (as each instruction will be at a different state).

- Read through the given modules to understand the code, fix/complete the code, and update/complete the block diagram RISC-V Stage 2.pptx to match your design, labeling the correct signal names and connecting or labeling the red control signals and the blue data registers.  Turn the completed diagram in via gradescope and copy to your repo (share your repo with your instructor).

  - Update pipeline_registers.vhdl to move key registers/signals from state to state.

  - Update riscv_pipeline.vhdl, removing syntax errors and ensuring correct operation of Stage 2. [many of these updates can be copied from riscv_multicycle.vhdl from stage 1]

  - Update instr_mem.vhdl, inserting the required NOPs (similar to RISC-V Exercise), but not extra NOPs, to ensure correct operation.

- Test Main Program: Run simulation in vivado, testing the main program works, including necessary signals, which as a minimum should include:

Clk, reset, clock_counter, pc, NPC, next_PC, if_id_npc, id_ex_npc, ex_mem_npc, instr, opcode, if_id_rs1, reg1_data, if_id_rs2, reg2_data, mem_wb_rd, wb_data, if_id_imm, alu_input_a,  alu_input_b, alu_result, mem_data, mem_read, mem_write, alu_src, id_ex_alu_src, branch, ex_mem_branch, jump, ex_mem_jump, load_addr, reg_write, reg_write_chip, display_x5, display_x6, display_x7, display_x10

Turn in your simulation plot in gradescope. Remember to annotate on the simulation plot the key points showing your simulation worked (like final clock cycle, final value of x5, x6, x7 and x10, etc)

- Answer the following questions in gradescope:

  - How many clock cycles to complete (end of first call to jump’s WB stage)?

  - What was the final value of register 5?

  - What was the final value of register 7?

  - How do the above answers compare to your expectations from the RISC-V exercise?

- No NOP Test: Now replace your instr_mem.vhdl with your original code (which does NOT have the NOPs) and run the simulation in vivado, including necessary signals, which as a minimum should include:

Clk, reset, clock_counter, pc, NPC, next_PC, if_id_npc, id_ex_npc, ex_mem_npc, instr, opcode, if_id_rs1, reg1_data, if_id_rs2, reg2_data, mem_wb_rd, wb_data, if_id_imm, alu_input_a,  alu_input_b, alu_result, mem_data, mem_read, mem_write, alu_src, id_ex_alu_src, branch, ex_mem_branch, jump, ex_mem_jump, load_addr, reg_write, reg_write_chip, display_x5, display_x6, display_x7, display_x10

  - Turn in your simulation plot in gradescope.

  - In gradescope,

    - What values does the PC have over time, and why?

    - What values does register 5 see, and what was the final value, and why?

    - What values does register 7 see, and what was the final value, and why?

    - Besides the above 3 issues, identify any other problems that occurred.

- Push all code to your github repo and share with your instructor [instr_mem.vhdl should be the version with the NOPs inserted]

Policy if Stage 1 not working

If your stage 1 code is not working, you can either attempt to get it working before implementing stage 2, or for a cost of 10 points of your stage 2 grade, your instructor can give you a working version of stage 1 to start from.

## Grading – Stage 2

| Item | Points | Out of |
| --- | --- | --- |
| Updated block diagram |  | 10 |
| Updated pipeline_registers.vhdl |  | 10 |
| Updated riscv_pipeline.vhdl |  | 15 |
| Update instr_mem.vhdl with NOPs inserted |  | 15 |
| Test Main Program |  | 20 |
| Questions in Gradescope on Main Program |  | 10 |
| Test and Question in Gradescope running original program without NOPs |  | 20 |
| Late Penalties (-25 points per day late) Instructor Provide Stage 1 code (-10 points) |  |  |
| Total |  | 100 |

GenAI and Collaboration Policy

For this assignment, use of generative AI (e.g., ChatGPT) or search engines is “level 2: Use of GenAI for brainstorming or idea generation.” For example, you can prompt GenAI on how to implement basic hardware in VHDL, such as “how to you implement a 32-bit register in VHDL?” “How to you implement a multiplexer using the when statement?”, or ask basic questions about the RISC-V Stage 2 pipeline architecture, like “why is the ALU controller separate from the Control Unit in RISC-V?”; however, any coding must be your own work.  Do not ask GenAI to produce specific code for this assignment, like “Given this VHDL code for the ALU, add the SUBI function.”

Any use of GenAI or internet searches must be clearly documented, including all prompts used. Failure to follow this policy will result in NO academic credit. One warning about GenAI and RISC-V: besides the common problem of GenAI hallucinations, there are different versions of RISC-V, different from this assignment, so GenAI could lead you astray.

The only collaboration allowed with classmates is for general questions, such has how to implement VHDL code, like “how to make a process statement to implement a register”, not for specific coding in this assignment, or for general concepts regarding the blocks in the block diagram.  All coding must be your own work.  Do not look at each other’s code.  You are not allowed to have another student look at your code to help with debugging, unless given specific authorization from your instructor.  All collaboration must be clearly documented; copying another person’s work, with or without documentation, will result in NO academic credit.